Precinct ___________________________

□ Record/Verify Precinct Name, Polling Place ID # and Hart serial #s on gray envelope

□ Place JBC on table and set-up an eSlate (extend legs & verify all parts are in working order)

□ Attach the gray cable to back of the JBC and power cord to JBC

□ Inspect eSlate booth rear port for bent/broken pins (if bent/broken contact supervisor)

□ Attach the gray (booth) cable from the JBC to the back of eSlate booth

□ Place TEST MODE election MBB Card into JBC

□ Power on JBC and reset JBC and eSlate

□ The following steps in Servo must be performed exactly to prevent the eSlate from locking-up:

- Select Admin Tools, device reset

- Connect the connecting cable to the JBC

- Select JBC/eSlate, OK

- Confirm the eSlate and JBC have been reset

- Watch for Servo to display “waiting for next device” & JBC/eSlate to display “device reset”.

- **WAIT 20 SECONDS** AFTER SEEING ABOVE MESSAGE

- Close out of Admin Tools

□ Power off JBC/eSlate

□ Take two battery packs for the JBC and test batteries

- Battery level should be 10.69 or greater (if under, write BAD on label)

- Fill-out and attach battery level label or write over old label, if room.

- Insert battery packs and connect batteries

□ Remove the eSlate from the booth. DO NOT LAY THE eSlate ON THE VBO SLIDER PLATE.

□ Take eSlate battery pack (1) and test.

- The battery level should be 10.69 or greater, (if under, write BAD on label).

- Fill-out and attach battery level label or write over old label, if room.

- Insert battery pack and connect battery.

□ Verify precinct on audio card matches precinct tag on eSlate handle and precinct name on JBC.

□ Install the Audio Card [verify card matches precinct on JBC/eSlate] (card should snap in)

□ Plug in the headphone jack

□ Reseat the eSlate in the booth. Make sure the eSlate sits on the posts and slides down firmly

□ Carefully remove the VBO from the eSlate Unit by disconnecting the power and data cables from                       back of the VBO

□ Remove the compartment cover from the VBO

□ Insert 6 batteries (AA) into battery pack and test VBO battery pack,

- Battery level should be 9 or greater, if not, replace batteries.

- Record battery level NEW & date_________.

- Reconnect battery pack.

□ Measure the paper roll with ruler:

- Make sure the roll is tightly wrapped

- Measure the feeder paper roll from the existing paper to the outside of the white plastic roller cap

- If the measurement is equal to, or greater than ONE CM, the feeder roll must be replaced

□ Write “L & A”, current date and your initials on paper roll

□ Attach the VBO cover

□ Attach the VBO connectors (power and data cables) and install cartridge

□ Verify “For VBO Only” label appears on the VBO power brick.

□ Plug in the small power cord from VBO power brick to the printer connection above VBO

□ Plug in the VBO power brick

□ Power on the JBC

□ VERIFY DATE & TIME ON JBC POWER UP REPORT

□ **REMINDER : DAYLIGHT SAVINGS WILL CAUSE JBC TO BE OFF 1 HOUR**

□ Enter PASSWORD in JBC and press ACCEPT

□ Enter POLLING PLACE  ID # (use PPID list) and press ACCEPT

□ Confirm Polling Place location using PPID list and info from JBC screen

□ Assign booth, Press ENTER button on the eSlate : “01” should be highlighted in yellow

□ Go to JBC and press DONE, then NEXT, and then select PRINT ZERO REPORT {if precinct is

located in a multiple, the other precincts will also appear on the zero report}

□ Open Polls so the eSlates are at the Access Code screen.

□ Add voter on JBC, get access code and press print.

□ Vote, using the hard copy test deck of ballots, test all keys and wheel to ensure eSlate and VBO

printers are functioning properly.

□ Check the audio by listening thru the headphones while you vote.

□ Verify votes by comparing electronic summary page on eSlate to printed ballot on VBO.

Note:  Voted test ballot remains on VBO paper roll to verify test was completed.

□ Repeat steps of adding voter/vote/verify (minimum of 3 test ballots)

□ If in a multiple precinct location, must “test” each precinct (see supervisor for details)

□ Press the Close Polls button on JBC

□ Confirm you are closing polls by pressing CONTINUE

□ Enter Password – press ACCEPT – PRINT TALLY {all precincts in that location will appear

on the tally}

□ Compare vote totals to the pre-determined totals report. If vote totals do not match take the eSlate and JBC out of service and notify a supervisor immediately.

□ Remove TEST MODE MBB Card from JBC

□ The following steps in Servo must be performed exactly to prevent the eSlate from locking-up:

- Select Admin Tools, device reset

- Connect the connecting cable to the JBC

- Select JBC/eSlate, OK

- Confirm the eSlate, JBC and MBB have been reset

- Watch for Servo to display “waiting for next device” & JBC/eSlate to display “device reset”.

- **WAIT 20 SECONDS** AFTER SEEING ABOVE MESSAGE

- Close out of Admin Tools

- Disconnect the connecting cable from the JBC

□ Power down JBC/eSlate/VBO

□ Disconnect JBC cable (gray) from the booth

□ Remove VBO by disconnecting the power and data cables

□ Remove compartment cover to VBO

□  Write “End L & A”, current date and your initials on paper roll

□ Advance paper till printout is not visible

□ Attach compartment cover to VBO

□ Record/Verify Blue Seal # on corresponding envelope

□ Apply Blue Seal to VBO (numbers facing up)

□ Attach VBO connectors (power and data cables) and install cartridge

□ Place headphone behind privacy screen

□ Disconnect VBO power cord from brick

□ Pack up VBO brick and power cord, making sure all cords are neatly tucked in and the door snaps closed

□ Make sure TEST MODE MBB Card has been removed from JBC

□ Clean eSlate screen with static wipe

□ Verify VBO seal has been attached and recorded

□ Verify the precinct name on the Election Mode MBB CARD matches the precinct card in the JBC window and the next lead precinct on your list

□ Write the MBB ID# on line provided _________________

□ Write the MBB ID# on gray envelope

□ Make sure Gray cable has been disconnected from eSlate/Make sure VBO is not plugged in

□ Insert  Election Mode MBB CARD in JBC

□ Power on JBC

□ Attach battery key to JBC

□ VERIFY THE CORRECT DATE AND TIME ON THE POWER-UP REPORT FROM THE JBC

□ **REMINDER : DAYLIGHT SAVINGS WILL CAUSE JBC TO BE OFF 1 HOUR**

□ Enter PASSWORD in JBC and press ACCEPT

□ Enter POLLING PLACE ID # from list (using PPID list)  and press ACCEPT

□ Confirm Polling Place Location using PPID list and info from JBC screen

□ Assign booth message will appear, press DONE, then NEXT

□ Print Zero Report

□ STOP! DO NOT OPEN POLLS.

□ Verify that the precincts reported on the zero report match the precincts listed on your polling place ID list and the label you attached to the JBC

□ Power down (disconnect the AC cable & battery key)

□ Reconnect only the battery key and press the keyboard combination “H-C-V” to turn off battery power.

□ Check the paper roll and replace if necessary.  Don’t forget to push the gray lever down!

□ Record/Verify Blue Seal # for JBC on gray envelope

□ Place Blue Seal on MBB door on JBC (numbers facing up)

□ Complete envelope

□ Place zero report in envelope

□ Cross-off polling place from list

□ Clean JBC with static wipes

□ Apply port blocker to open printer port on back of JBC

□ Tape “Election Morning” notice on JBC

□ Tape “eSlate” notice to eSlate screen

□ Close-up eSlate booth & attach security seal (Zip Tie Seal) and record/verify # on envelope

□ Leave cables attached and place JBC in box

□ Place “Open & Close Polls” instructions on top of JBC in box

□ Take down eSlate booth

_________________________	__________		_________________________	__________

Election Official			Date			Election Official			Date

SUPPLY LIST

- Preparation worksheets

- Labeled envelopes to hold checklist

- Polling Place ID list with address

- JBC Paper rolls

- Tamper evident seals [may be in gray envelope]

- Battery Packs

- Open/Close Polls Step By Step Instructions

- JBC STOP instruction sheet

- Audio Cards

- Precinct List

- Trays for completed worksheets

- VBO power brick labels

- VBO paper rolls

- VBO seals [may be in gray envelope]

- eSlate seals [may be in gray envelope]

- Pens

- Power strips

- Scissors

- Battery labels

- Static wipes

- Rulers